ParkZone F-27Q Stryker w/ speed brakes DX7s Setup

DISCLAIMER 
Attention- Safety First!

The model setup files provided here can greatly simplify setting up complex programming in your DX7s for your favorite aircraft. Because mechanical setups can vary on some types of models, it is your responsibility to ensure all controls are operating in the proper direction and within the manufacturers recommended throws. You should also take time to understand the anticipated flight characteristics for each flight mode that might be enabled.

Plug left rudder into the Aux 1 channel. When you turn on you will find a warning for the throttle if it is above 10% and the flap switch if in norm or landing position. Use the flight mode switch to control the speed brakes. You can use the flap switch if you wish, just change the setting in the flap menu to reflect this. With the flight mode switch in position 1 this is normal flight position (no speed brakes). Move the flight mode switch to position 0 and the rudders will deflect inboard and the elevons will deflect down a small amount. Move the flight mode switch to position 2 and the rudders will deflect outboard and the elevons will deflect down a small amount. Both settings slow the model down and work well.

Works with files:

PKZ5675
PKZ5680